Form B/Form BUZ RTD temperature sensor with Flanged（TM1505)

Applications:

HVAC,Refrigeration, Ovens, Mechanical equipments, Petrochemical industries, Metallurgy, and Electric power etc.

Technical data:

Protection tube：stainless steel 304/316 or Abrasion resistance material

Sensor element:  Pt100, Pt500, Pt1000 meets IEC 751

Tolerance :     Class B: △t=±(0.30 + 0.005 |t| )℃(standard)

Class A: △t=±(0.15 + 0.002 |t| )℃

Protection Class： IP65

Integrated transmitter:Output 4-20mA, 2-wire Power supply: 12-30VDC

Load resistance 0-600Ω/24V Accuracy:±0.5%FS，±0.2%FS

Response time：t0.9  approx. 50secs, in water 0.2 m/sec, diameter 6mm

接线图-Wiring diagram 铂电阻RTD

单支2线 1x2-wire单支3线 1x3-wire单支4线 1x4-wire

双支4线 2x4-wire双支6线 2x6-wire

红色标记 red mark安装螺钉 mounting screw

Dimensions（外形尺寸图）

Order Details:

| TM1505 | — | (1) | — | (2) | — | （3） | — | (4) | — | (5) | — | (6) | / | (7) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- (1)protection tube diameter D

9       9mm

11      11mm

96       9mm stepped down to 6mm

116      11mm stepped down to 6mm

- （2）Insertion length EL

- 160     160mm

- 200     200mm

Note: other lengths as per customers’ requirements(EL≤1000mm）

- Process connection G

- 310   flexible flange D70-9

- 311    flexible flange D70-11

- 312   sleeve flange  D60-8

- 313   sleeve flange  D60-6

- 314   sleeve flange D50-4

- 315   sleeve flange  D50-3

- 316   sleeve flange  D70-12

- 317   sleeve flange  D70-16

- 318  sleeve flange  D70-20

- 330    HG20592RF PN1.0 DN25

- 340    HG20592RF PN1.0 DN40

- 350    HG20592RF PN1.0 DN10

- Details in “Thread code table”

- Sensor element

13     1 x Pt100 in 3-wire circuit

53    1 x Pt500 in 3-wire circuit

63    1 x Pt1000 in 3-wire circuit

16     2x Pt100 in 6-wire circuit

56     2 x Pt500 in6-wire circuit

66     2x Pt1000 in 6-wire circuit

14     1 x Pt100 in 4-wire circuit

(5)Measuring range

20    -50℃ ～+ 400℃

21     -50℃ ~+600℃

25    -200 ℃~+400℃

(6) Cable length L（L≥100mm）

100     100mm

200     200mm

2000    2000mm

Note: other lengths as per customers’ requirements

(100mm≤L<1000mm, L1=50mm, L≥1000mm, L1=100mm)

(7)Additional options

00      No

02      shielded connecting cable

06      Output 4-20mA in 2-wire

07      Ferrules as per DIN 46 228 part 4(standard)

08      Multipole connector (providing types)

99      Special options

List the additional options in sequence, separated by commas.